Name: Joshith Allen M

Matriculation No: 00164773

Intelligent Robotics

Task 1 :  Parking Assistant

1.Introduction

First, Complete code is written in Java for this Task. The task describes a driver assistance system that look for a parking space while driving along a straight path. While searching, the vehicle must satisfy certain criteria:

- Drive parallel along the road on the right-hand side.

- While driving the robot maintains a distance of 60 cm from the right-hand side.

- The robot should measure parking space on the right-hand side while driving.

- If the space is too small, continue searching for parking space.

- If the space is large enough, check if parking is permitted based on signs.

- If no parking signs allow parking, if not don’t park.

- Detect obstacles in the driving, the robot must not drive around them rather wait until the obstacle is cleared.

2. Overview of Robot Behavior.

Explaining robot behavior in the environment.

- Start driving forward at a constant speed.

- Front Ultrasonic Sensor continuously checks for obstacles in the driveway area in the front.

- Right Ultrasonic Sensor continuously checks to maintain lateral distance of 60 cm.

- Right Ultrasonic Sensor also checks for parking space while moving.

- Detects a parking gap when the side distance suddenly becomes large.

- Simultaneously the camera checks for no parking signs while a gap is detected.

- When the gap ends :

- If the measured length is enough and no parking signs, perform parking. Parking is done in reverse.

- Otherwise continue searching

3. Sensor Interface and Control Parameters

3.1 Sensors Used.

- Front Ultrasonic Sensor: Sensor Number 4

Used for detecting obstacle in the driveway.

- Side Ultrasonic Sensor: Sensor Number 7

Used for maintaining 60 cm clearance on the right-hand side

Used for measuring the start and end of a parking slot.

- Camera: Uses blob detection.

This gets red and blue blobs X coordinates.

If either return >=0, then a blob is detected (no parking detected).

This is a simple color presence check rather than check for full sign classification.

3.2 Control Parameters.

These constants define the behaviour of the system:

All distances are in mm

3.2.1 Clearance control :

- SIDE_TARGET_DISTANCE = 600

The desired distance from the right-hand side 60 cm.

- SIDE_DISTANCE_MARGIN = 50

This error tolerance. Between 550 and 650 mm the system will not correct.

3.2.2 Thresholds for Parking Space detection :

- SLOT_OPEN_LIMIT = 1000

If side distance is above 1000, the robot interprets it as open space for parking

- SLOT_CLOSE_LIMIT = 800

If side distance below this during scanning, the robot treats it as there is no parking space.

- OPEN_CONFIRM_HITS = 2

The robot requires the open condition to be detected twice in a row to confirm a real gap. This means that only after getting two times in a row the robot confirms there is a space for parking.

3.2.3 Minimum Gap for parking space :

- REQUIRED_SLOT_SIZE = 900

If measured gap length is below this, it is rejected.

3.2.4 Distance for Obstacle stopping in front :

- FRONT_BRAKE_RANGE = 400

If the front sensor reads less than this, the vehicle stops and waits.

3.2.5 Cruise speed :

- CRUISING_SPEED = 150

The constant speed used while searching. This is also used for slot length estimation.

3.2.6 Camera orientation:

- panTilt(-90, -5)

This rotates camera 90 degrees to the right-hand side.

4. Control Logic for the Robot. Brief Overview

The robot runs in a while loop.

For each iteration:

  - Computes deltaSeconds from the system clock.

- This is the time between the loop cycles.

- We compute deltaSeconds so that slot length measurement is independent of loop frequency. Since the loop execution time varies, using real elapsed time ensures accurate distance estimation through speed × time.

  - Reads Sensors and blobs :

Reads front and side ultrasonic sensors and the blob values

- frontRange = getSonarRange(FRONT_ULTRASONIC)

- sideRange = getSonarRange(SIDE_ULTRASONIC)

- redBlobX = getBlobX(1)

- blueBlobX = getBlobX(2)

- signVisible = (redBlobX >= 0 || blueBlobX >= 0)

  - Runs if conditions to check :

- For Obstacle in front.

- To Maintain a certain distance in right-hand side.

- Logic for Tracking and Scanning :

- Tracking means the robot is driving beside right-hand side and looking for parking space.

- Scanning means the robot is in parking space and measuring it’s length.

5. Obstacle Detections in Front on the Driveway.

“if” condition is implemented such that if frontRange is within the brake range (< 400)

- Stop the vehicle as speed become 0 : speed(0)

- After that continuously checks until distance increases above a safe threshold : FRONT_BRAKE_RANGE + 100

- If above the safe threshold the robot resume cruising.

- Here the robot does not travel around the obstacle rather waits until the obstacle is cleared.

6.  Maintaining 60 cm clearance.

6.1 Steering update.

- Instead of steering every loop, your  code updates steering every 200 ms (STEERING_INTERVAL_MS).

- This means every 0.2 seconds it checks the side distance and corrects if needed.

6.2 Control Method.

- Calls method holdSideClearance with the inputs of sideRange from Ultrasonic sensor, SIDE_TARGET_DISTANCE of 60 cm and SIDE_DISTANCE_MARGIN as an error tolerance.

holdSideClearance(sideRange, SIDE_TARGET_DISTANCE, SIDE_DISTANCE_MARGIN)

In holdSideClearance method :

- If the robot is far, it moves closer.

- If the robot is close,  it moves away.

7. Parking Space Detection  and Measurement.

  - Tracking Mode.

In Tracking the robot starts to detect the gap :

- If sideRange >= SLOT_OPEN_LIMIT (≥ 1000 mm), it increments openHitCounter.

- Otherwise, it  resets to 0

If openHitCounter >= OPEN_CONFIRM_HITS

- It confirms a real gap.

- Switches to MODE_SCANNING.

- Increments slot counter for detecting. This is number of parking space detected so far.

- After this resets measurement variables:

slotDistance = 0

parkingForbiddenDetected = false.

  - Scanning Mode.

In Scanning  the robot starts to measure the gap :

    - Parking Space length Calculation :

- The space is calculated by common formula : distance = speed x time

slotDistance += CRUISING_SPEED * deltaSeconds

    - Detection of No Parking sign :

- While scanning, if signVisible becomes true, eventually parkingForbiddenDetected = true. Meaning that no parking sign is detected during the available parking space.

  - Detection at End of the parking space :

- This is done to evaluate the detected parking space has ended or not. Once the robot detects the parking space is close, the robot stops.

sideRange <= SLOT_CLOSE_LIMIT

- Now the robot prints the parking space number, the measured value and forbidden status of parking.

- After this, it checks if slotDistance >= REQUIRED_SLOT_SIZE and parkingForbiddenDetected == false.  The robot performs parking routine.

- Else, the robot returns to tracking mode.

8. Performing Parking.

If a suitable parking space is found, the function performs the method executeParkingRoutine().

- This method runs a fixed move based on the current environment.

- It is not a dynamic planner, but a pre-planned motion.

9. Conclusion

- The robot execution is planned as two modes :

a. Tracking :  Drive forward parallel to right-hand side and maintain distance of 60 cm as clearance.

b. Scanning : Detect a parking space, measure it’s length and decide whether to park.

- The side ultrasonic sensor is used to:

a.  Keep the robot at a certain distance from the right-hand side.

b. To detect start and end of a parking space using distance thresholds.

-  The parking space length is calculated by formula distance = speed x time, which keeps measurement stable even when loop timing varies.

- The front ultrasonic sensor ensures safety by:

a.  Stopping the robot when an obstacle is within the braking range.

b.  No overtaking rather wait until the obstacle is cleared in driveway area.

- If the slot is large enough and not restricted, the robot performs a predefined reverse parking routine to park the vehicle.

- The no parking detection is implemented using camera blob visibility, if either red or blue is detected it is concluded that the parking space is restricted.